Dichromate Bath

Magnesium castings require protection from atmospheric attack; the factory soaked the castings in a Dichromate bath.  The castings were then painted silver for hubs and brake-plates and matt black for engine parts.

The constituents required for a one-gallon bath are:

- 1 gallon water				stock code

- 500 gramme Sodium dichromate		301304M

- 250 gramme Magnesium sulphate		291172L

- 250 gramme Manganese (2) sulphate	291474W

The stock codes are for owners in the UK where the chemicals can be obtained from Analytical Supplies – Tel No: 01332-831671.

Apparently the parts are submerged in the solution for 24/48 hours.